RASPBERRY PI + GITHUB ACTIONS

Pico & STM32F446RE Firmware CI/CD Guide

From a blank Raspberry Pi to hands-free flashing on every push — covers hardware setup, tool installation, self-hosted GitHub Actions, VS Code project setup, and the day-to-day Git workflow.

| Field | Value |
| --- | --- |
| GitHub repository | dprime999/HILTester |
| Raspberry Pi login | dprime999@raspberrypi |
| Local environment | VS Code and Git (Windows/macOS/Linux) |
| Target hardware | Raspberry Pi Pico + STM32F446RE (Nucleo) |
| Purpose | Push firmware source and have a self-hosted GitHub Actions runner build and flash both boards automatically |

SCOPE This guide walks through the full pipeline end to end: preparing the Pico for unattended (no-hands) flashing, installing the build toolchain on the Pi, wiring up SSH and a self-hosted GitHub Actions runner, setting up the local project in VS Code, and the everyday commit-and-flash workflow. Exact paths and board names should be adapted if your repo or hardware differs.

Prepared August 16, 2026

# 1. What the Pipeline Does

The setup has three moving parts: GitHub stores the project and workflow definition, the Raspberry Pi runs jobs as a self-hosted GitHub Actions runner, and your PC (via VS Code) is where you write and push firmware source code. Nothing gets built on your PC — the Pi builds both projects itself and flashes the hardware connected to it.

| Component | Responsibility |
| --- | --- |
| VS Code / PC | Edit blink.c / STM32 source, commit, and push to GitHub. No local build required. |
| GitHub | Stores the repo and workflow file. Queues a job on push to main. |
| Raspberry Pi (runner) | Checks out the repo, builds both projects with CMake, then flashes the Pico with picotool and the STM32F446RE with openocd. |

IMPORTANT A self-hosted runner executes repository workflow code on your Raspberry Pi. Use it only with repositories and contributors you trust. Avoid exposing an unreviewed public repository to a privileged runner.

# 2. Prerequisites

Before starting, make sure you have access to the following.

## Accounts and software

- A GitHub account with write access to the target repository (e.g. dprime999/HILTester).

- VS Code installed on your PC, with Git available on the command line.

- An SSH client (built into VS Code's terminal, PowerShell, or Terminal on macOS/Linux).

## Hardware

- A Raspberry Pi (Pi 5 or similar) powered on and connected to the same network as your PC.

- A Raspberry Pi Pico, connected to the Pi via USB.

- An STM32F446RE board (Nucleo), connected to the Pi via its onboard ST-Link USB.

- Both boards should be plugged into the Pi itself — the Pi is what runs the build and flash steps, not your PC.

A good first checkpoint before touching any config: confirm you can open VS Code, sign in to GitHub in the browser, and physically see the Pi, Pico, and STM32 board on your desk. Everything below builds on top of that.

# 3. Hardware Setup: Pico BOOTSEL and Unattended Flashing

The Pico only accepts new firmware while it's in BOOTSEL mode, which normally requires holding the BOOTSEL button while plugging in the USB cable. For a CI pipeline that flashes on every push, you want to skip that manual step after the very first flash.

## 3.1 Find the Pico on the Pi

From an SSH session on the Pi, check whether the board is visible over USB:

lsusb

What to look for:

- In BOOTSEL mode, the Pico shows up as something like 2e8a:0003 Raspberry Pi RP2 Boot — this means it's mounted as a mass-storage device and ready to accept a .uf2 file.

- Running normal firmware with USB serial enabled, it may instead show up as a CDC/ACM serial device.

- If it doesn't appear at all, it's either unpowered, on a data-incapable USB port/cable, or running firmware with no USB support — in which case the very first flash needs the manual BOOTSEL process below.

## 3.2 First flash (manual BOOTSEL)

The very first firmware you put on a fresh Pico has to be done by hand, since it can't yet respond to software-triggered reboots:

- Unplug the Pico.

- Hold down the BOOTSEL button on the board.

- While still holding BOOTSEL, plug the USB cable back in.

- Release the button once it's connected — the Pico will mount as a USB drive named RPI-RP2.

- Copy the .uf2 file onto that drive (or use picotool load -f -v path/to.uf2, which also works at this stage).

## 3.3 Enable no-hands re-flashing going forward

To let picotool force the board back into BOOTSEL mode over USB — no button press needed — the firmware itself needs to expose a USB stdio interface. Two changes make this permanent for every future build:

- In CMakeLists.txt, enable USB stdio for the target:

pico_enable_stdio_usb(blink 1) pico_enable_stdio_usb(blink 1) # replace 'blink' with your target name

- In main.c / blink.c, call stdio_init_all() early in main() so the USB interface actually initializes.

Once a build with these two changes has been flashed at least once (using the manual BOOTSEL steps above), every subsequent flash can use:

picotool load -f -v /path/to/blink.uf2

The -f flag forces the currently-running Pico into BOOTSEL mode automatically, loads the new firmware, and the board resumes running — no unplugging, no button presses. This is exactly what the "Flash Raspberry Pi Pico first" step in the workflow relies on.

IF -f STOPS WORKING If picotool load -f suddenly can't find the device, run lsusb first to confirm the Pico is even enumerating. If it's missing entirely, check the cable/port, then fall back to a single manual BOOTSEL flash (3.2) to restore the USB stdio firmware, and no-hands flashing will resume working from there.

Reference: https://www.eldelto.net/articles/raspberry-pi-pico-no-hands-flashing

# 4. Install Build and Flashing Tools on the Raspberry Pi

SSH into the Pi (see Section 6 if SSH isn't set up yet) and install the compiler toolchain and build tools:

sudo apt update sudo apt install -y build-essential cmake make git \ gcc-arm-none-eabi libnewlib-arm-none-eabi libstdc++-arm-none-eabi

| Package | Why it's needed |
| --- | --- |
| build-essential, make | General compiling tools (gcc for the host, make). |
| cmake | Build-system generator used by both the Pico SDK and STM32 CubeMX projects. |
| git | Needed for checkout and for the Pico SDK's own submodules. |
| gcc-arm-none-eabi | Cross-compiler that targets the Cortex-M cores in both the RP2040 (Pico) and STM32F446 (Nucleo). |
| libnewlib / libstdc++-arm-none-eabi | C/C++ standard library builds for the bare-metal ARM target. |

You'll also need openocd to flash the STM32 board, and picotool to flash the Pico:

sudo apt install -y openocd # picotool is installed via the Pico setup script below

## 4.1 Install the Pico SDK

Run this from the Pi's home directory (or wherever you keep SDK-related tooling). This script installs the Pico SDK, examples, and picotool:

cd ~ sudo apt install wget wget https://raw.githubusercontent.com/raspberrypi/pico-setup/master/pico_setup.sh chmod +x pico_setup.sh ./pico_setup.sh sudo reboot

After the reboot, confirm everything landed correctly:

arm-none-eabi-gcc --version cmake --version openocd --version picotool version

PICO_SDK_PATH Note where pico_setup.sh placed the SDK (commonly ~/pico/pico-sdk). Your GitHub Actions workflow needs this exact path set as the PICO_SDK_PATH environment variable — see Section 10.

ST-LINK USB PERMISSIONS openocd sometimes can't claim the ST-Link USB device without root unless udev rules are installed. If flashing the STM32 fails with a permissions/USB claim error, install openocd's udev rules (commonly at contrib/60-openocd.rules in its source, or via your package's postinst) and add your user to the plugdev group, then re-login.

# 5. Connect to the Raspberry Pi by SSH

All runner setup and tool installation happens on the Pi itself. From PowerShell, Terminal, or the VS Code integrated terminal:

ssh dprime999@raspberrypi

If the hostname doesn't resolve, use the Pi's IP address instead:

ssh dprime999@192.168.x.x

- First connection: type yes to accept the host key, then enter the Pi account password.

- Connection refused: confirm SSH is enabled on the Pi and both machines are on the same network.

- Permission denied: double-check the username and password.

## 5.1 Reset or change the Raspberry Pi password

To change the current user's own password (you'll be prompted for the current password, then the new one twice):

passwd

From another administrator account with sudo access, reset the dprime999 password directly:

sudo passwd dprime999

FORGOTTEN PASSWORD If no working administrator account is available, recovery requires local access to the Pi or its storage media and depends on the installed OS. Don't repeatedly guess credentials over SSH — use the OS-specific recovery procedure or re-image the device if needed.

# 6. Register the Raspberry Pi as a Self-Hosted Runner

## 6.1 Open the runner setup page on GitHub

- Open the repository on GitHub.

- Go to Settings → Actions → Runners → New self-hosted runner.

- Select Linux and the architecture matching the Pi OS (ARM64 for a 64-bit Pi OS install).

## 6.2 Download and extract the runner

Keep the GitHub setup page open — it generates current download commands and a short-lived registration token. Copy those exactly rather than reusing old ones. The commands look structurally like this (versions/hashes change):

mkdir -p ~/actions-runner && cd ~/actions-runner curl -o actions-runner-linux-arm64-<version>.tar.gz -L \ https://github.com/actions/runner/releases/download/v<version>/actions-runner-linux-arm64-<version>.tar.gz tar xzf ./actions-runner-linux-arm64-<version>.tar.gz

## 6.3 Configure the runner

./config.sh --url https://github.com/dprime999/HILTester --token <TOKEN_FROM_GITHUB>

Accept or customize the prompts for runner group, name, labels, and work folder. A practical runner name is raspberrypi. The token expires quickly — regenerate it from the GitHub page if configuration fails.

## 6.4 Test interactively, then install as a service

cd ~/actions-runner ./run.sh

The terminal should report it's connected and listening for jobs. Push a test commit or manually dispatch the workflow from the Actions tab, then Ctrl+C once confirmed.

sudo ./svc.sh install sudo ./svc.sh start sudo ./svc.sh status

Other useful commands:

sudo ./svc.sh stop sudo ./svc.sh start sudo ./svc.sh status sudo ./svc.sh uninstall

WORKING DIRECTORY Run svc.sh from inside the actions-runner directory (cd ~/actions-runner first) or the command won't be found.

Once registered and running as a service, this is the one folder on the Pi that should survive any future cleanup — everything else (build output, old firmware, scratch folders) can be wiped and regenerated, but actions-runner/ holds your runner's identity and registration.

# 7. Set Up the Project in VS Code

## 7.1 Install extensions

- Raspberry Pi Pico extension — project scaffolding and SDK integration for Pico/RP2040 projects.

- STM32 for VS Code (or use STM32CubeMX/CubeIDE separately) — for generating and configuring the STM32F446RE project, pin mux, and clock config.

- C/C++ extension and CMake Tools — IntelliSense and CMake integration for both projects.

If you're generating the STM32 project from scratch, do that in CubeMX first (peripheral/pin configuration, then "Generate Code" targeting a CMake-based toolchain) before opening the generated folder in VS Code.

## 7.2 Clone the repository

cd C:\path\to\your\workspace git clone https://github.com/dprime999/HILTester.git cd HILTester

## 7.3 Write the firmware — don't build locally yet

With the repo cloned, write or copy in your source: main.c / blink.c for the Pico project and the STM32 sources (Core/, Drivers/, the .ioc file, linker script, startup file) for the STM32 project. At this stage, just get the code in place — the Pi's Actions runner is what builds it, so there's no need to build locally first. Arrange the two projects as siblings at the repo root:

HILTester/ .github/workflows/main.yml .gitignore blink/ (Pico project) CMakeLists.txt blink.c pico_sdk_import.cmake blink2/ (STM32 project) CMakeLists.txt blink2.ioc Core/ Drivers/ cmake/ startup_stm32f446xx.s STM32F446xx_FLASH.ld

DON'T COMMIT build/ Neither project's build/ directory should ever be pushed. It's regenerated fresh by the runner on every push — see Section 9 for the .gitignore that enforces this.

# 8. Git Workflow: Add, Commit, Push, and Sync

## 8.1 Review, commit, and push

git status git add blink blink2 .gitignore git commit -m "Add blink and blink2 project source" git push origin main

After the push, open the GitHub Actions tab and watch the job run on the Pi.

## 8.2 When GitHub rejects the push: fetch and rebase

If a commit was made on GitHub directly (e.g. an edit through the web UI) while you also had local commits, histories diverge and the push is rejected with a "fetch first" or non-fast-forward message.

git fetch origin git status git pull --rebase origin main git push origin main

| Command | What it does |
| --- | --- |
| git fetch origin | Downloads updated remote history and refreshes origin/main without touching your working files. |
| git pull --rebase origin main | Fetches remote commits and replays your local commits on top of them. |
| git push origin main | Uploads the now-linear history to GitHub. |

### Resolving a rebase conflict

Open the affected file in VS Code, choose or combine the correct changes, save, then continue:

git add <resolved-file> git rebase --continue git push origin main

To cancel the rebase and return to the pre-rebase state:

git rebase --abort

DON'T FORCE-PUSH CASUALLY Avoid git push --force on main — it can overwrite commits already on GitHub. Pull with rebase and resolve history unless you specifically intend to rewrite it.

## 8.3 Day-to-day routine

git pull --rebase origin main # edit blink.c / STM32 source files git status git add blink blink2 git commit -m "Update firmware" git push origin main # watch the Actions tab — the Pi builds and flashes both boards automatically

# 9. The .gitignore File

Placed at the repo root (alongside blink/ and blink2/), this keeps every generated build artifact — CMake caches, Ninja logs, compiled .elf/.uf2 files — out of version control entirely, since the runner regenerates all of it on every push.

# Build output (regenerated by CI on every push — never commit this) build/ build*/ **/build/ # CMake cache/config leftovers CMakeCache.txt CMakeFiles/ cmake_install.cmake CTestTestfile.cmake compile_commands.json # Ninja *.ninja_log *.ninja_deps build.ninja # Editor / OS junk .vscode/ .clangd .cache/ *.log .DS_Store Thumbs.db

# 10. Sample GitHub Actions Workflow

Save this as .github/workflows/main.yml. It checks out the repo, verifies tools are present, builds both projects, flashes the Pico first and the STM32F446RE second, then uploads the compiled binaries as a downloadable artifact.

name: Flash Pico and STM32F4 on: push: branches: [ "main" ] workflow_dispatch: permissions: contents: read concurrency: group: hil-hardware-flashing cancel-in-progress: false jobs: flash-hardware: name: Flash Pico then STM32F4 runs-on: self-hosted timeout-minutes: 15 env: PICO_SDK_PATH: /home/dprime999/pico/pico/pico-sdk steps: - name: Check out pushed files uses: actions/checkout@v6 - name: Verify required source files and tools shell: bash run: ¦ set -euo pipefail test -f "${GITHUB_WORKSPACE}/blink/blink.c" test -f "${GITHUB_WORKSPACE}/blink2/CMakeLists.txt" command -v picotool command -v openocd - name: Configure and build Pico project shell: bash run: ¦ set -euo pipefail cmake -S "${GITHUB_WORKSPACE}/blink" -B "${GITHUB_WORKSPACE}/blink/build" cmake --build "${GITHUB_WORKSPACE}/blink/build" -j"$(nproc)" - name: Configure and build STM32 project shell: bash run: ¦ set -euo pipefail TOOLCHAIN="${GITHUB_WORKSPACE}/blink2/cmake/gcc-arm-none-eabi.cmake" cmake -S "${GITHUB_WORKSPACE}/blink2" \ -B "${GITHUB_WORKSPACE}/blink2/build" \ -DCMAKE_TOOLCHAIN_FILE="$TOOLCHAIN" cmake --build "${GITHUB_WORKSPACE}/blink2/build" -j"$(nproc)" - name: Flash Raspberry Pi Pico first shell: bash run: ¦ set -euo pipefail picotool load -f -v "${GITHUB_WORKSPACE}/blink/build/blink.uf2" - name: Wait for USB devices to settle shell: bash run: sleep 3 - name: Flash STM32F4 second shell: bash run: ¦ set -euo pipefail openocd \ -f interface/stlink.cfg \ -f target/stm32f4x.cfg \ -c "program ${GITHUB_WORKSPACE}/blink2/build/blink2.elf verify reset exit" - name: Upload All Compiled Firmware to GitHub uses: actions/upload-artifact@v4 with: name: compiled-mcu-firmware path: ¦ blink/build/blink.uf2 blink2/build/blink2.elf

YAML NOTE YAML is indentation-sensitive — use spaces, never tabs. The runs-on label must match the label(s) shown for your registered runner in GitHub; if GitHub shows different labels, update the workflow accordingly.

# 11. How Actions, the _work Folder, and Artifacts Fit Together

It helps to know what's actually happening on disk on the Pi when a workflow runs.

## 11.1 The runner's folder layout

~/actions-runner/ _work/ HILTester/ HILTester/ <- your repo gets checked out here (this is $GITHUB_WORKSPACE) bin/ externals/ run.sh config.sh svc.sh

Every workflow run gets its source checked out into _work/<repo>/<repo>, exposed inside the workflow as the $GITHUB_WORKSPACE environment variable. That's why the sample workflow references paths like ${GITHUB_WORKSPACE}/blink instead of a hardcoded path — it works no matter where actions-runner itself lives.

## 11.2 Why the workspace is always clean

The checkout action runs a git clean before pulling the latest commit, which removes anything not tracked by git — including old build/ folders left over from a previous run. That's what guarantees every build starts fresh rather than accumulating stale artifacts over time.

## 11.3 What "artifacts" means in this context

The final step, actions/upload-artifact, packages the specified files (the compiled .uf2 and .elf) and attaches them to that specific workflow run in GitHub. After a run finishes, they're downloadable as a zip from the run's summary page on the Actions tab — useful for grabbing a copy of exactly what was flashed without needing to rebuild it yourself.

## 11.4 Why the concurrency block matters

The concurrency group at the top of the workflow ensures that if two pushes happen close together, the second run waits for the first to finish rather than both trying to flash the same USB-connected boards at once.

# 12. Current Day-to-Day CI/CD Workflow

Once everything above is set up, the actual routine for shipping new firmware is short:

- Clone the repo (or pull the latest changes if already cloned).

- Edit the .c source files for the Pico and/or STM32 project in VS Code.

- Add, commit, and push — all from within VS Code or its integrated terminal.

- Wait roughly 20 seconds. The self-hosted runner on the Pi picks up the push, builds both projects, flashes the Pico first, then the STM32F446RE.

- Check the Actions tab to confirm the run succeeded and, optionally, download the compiled-mcu-firmware artifact.

# 13. Verification Checklist

☐  Repository contains .github/workflows/main.yml and a root-level .gitignore.

☐  blink/ and blink2/ project source sit at the repo root with no build/ directory committed.

☐  Raspberry Pi appears under Settings → Actions → Runners and shows Idle or Active.

☐  Interactive ./run.sh test received a job successfully before installing the service.

☐  Runner service is installed, started, and reports an active status.

☐  lsusb on the Pi shows the Pico, and picotool load -f successfully re-flashes it without a manual BOOTSEL press.

☐  openocd can see and program the STM32F446RE over its ST-Link interface.

☐  PICO_SDK_PATH in the workflow matches the actual SDK location on the Pi.

☐  A push to main triggers the workflow and it completes successfully end to end.

☐  Remote edits are integrated with git pull --rebase before the next push.

# 14. Quick Troubleshooting

| Symptom | Recommended check |
| --- | --- |
| Runner stays Offline | SSH to the Pi, run cd ~/actions-runner && sudo ./svc.sh status. Start it with sudo ./svc.sh start. |
| Job waits for a runner | Confirm runs-on labels match the registered runner's labels and that it's online. |
| Pico not found by picotool / lsusb | Check the cable and port. If the board was never flashed with USB-stdio firmware, do one manual BOOTSEL flash (Section 3.2) to restore no-hands flashing. |
| openocd can't claim the ST-Link device | Check udev rules and that your user is in the plugdev group; avoid running the workflow step with sudo. |
| No .uf2 / .elf found after build | Confirm CMake configured successfully and the toolchain file path for the STM32 project is correct. |
| Push rejected: fetch first | Run git fetch origin, then git pull --rebase origin main, resolve conflicts, and push again. |
| Invalid username or token over HTTPS | Use Git Credential Manager or browser sign-in — GitHub account passwords aren't accepted for Git pushes. |

# References

No-hands Pico flashing (picotool -f): https://www.eldelto.net/articles/raspberry-pi-pico-no-hands-flashing

Original CI/CD tutorial basis: https://medium.com/into-the-ai/raspberry-pi-a-web-server-with-ci-cd-pipeline-fd077b3be63a

GitHub Docs — runner service commands: https://docs.github.com/en/actions/how-tos/manage-runners/self-hosted-runners/configure-the-application

Raspberry Pi Documentation — remote access (SSH): https://www.raspberrypi.com/documentation/computers/remote-access.html

Pico setup script: https://raw.githubusercontent.com/raspberrypi/pico-setup/master/pico_setup.sh

picotool repository: https://github.com/raspberrypi/picotool

VERSIONS CHANGE GitHub's runner download version, checksums, architecture options, and registration tokens are generated dynamically. Always use the exact commands shown on the repository's current New self-hosted runner page rather than reusing old ones from notes.